WT588D CHIPS & MODULES DETAILED INFORMATION

Catalog

1.FEATURES	3

2. WT588D FUNCTION DESCRIPTIONS	4

3.WT588D SELECTION	4

4.THE RELATION BETWEEN FLASH CAPACITY AND DURATION	4

5.APPLICATIONS	5

6.WT588D APPLICATION DIAGRAM	5

7.PACKAGE SKETCH	6

8.PIN DESCRIPTIONS	6

8.1.  16PIN MODULE PIN’S DESCRIPTION	6

8.2. 28PIN MODULE(V1.1) PIN’S DESCRIPTION	7

8.3. 28PIN MODULE(V1.2) PIN’S DESCRIPTION	8

8.4. LQFP32 PACKAGE PIN’S DESCRIPTION	9

8.5.DIP18 PACKAGE PIN’S DESCRIPTION	10

8.6.SSOP20 PACKAGE PIN’S DESCRIPTION	10

9.PIN’S DETAILED DESCRIPTION	11

10.ELECTRONIC PARAMETERS	11

11.ENVIRONMENTAL LIMITED PARAMETER	12

12.CONTROL MODE	12

12.1. MP3 MODE	12

12.2. KEY CONTROL MODE	12

12.2.1. EDGE RETRIGGER	13

12.2.2. EDGE NO RETRIGGER	13

12.2.3. LEVEL LOOP	13

12.2.4. LEVEL UNLOOP	13

12.2.5. LEVEL HOLD LOOP	14

12.2.6. ON/OFF	14

12.2.7.NEXT UNLOOP	14

12.2.8. PREV UNLOOP	14

12.2.9. NEXT LOOP	15

12.2.10.PREV LOOP	15

12.2.11. PAUSE	15

12.2.12. STOP	15

12.3. 3X8 MATRIX KEY CONTROL MODE	15

12.4.PARALLEL CONTRL MODE	16

12.4.1. SPEECHES AND ADDRESSES CORRESPONDANCE	16

12.4.2 CONTROL TIMING DIAGRAM	16

12.5.ONE-LINE CONTROL MODE	16

12.5.1. ASSIGNMENT OF I/O	16

12.5.2.COMMAND AND CODE	17

12.5.3.THE RELATIONSHIP BETWEEN SPEECH AND ADDRESS	17

12.5.4.CONTROL TIMING DIAGRAM	17

12.2.5. ONE LINE SERIAL CONTROL DIAGRAM SAMPLE	18

12.5.5. PROGRAM EXAMPLE	18

12.6.THREE-LINE SERIAL CONTROL MODE	18

12.6.1.ASSIGNMENT OF I/O	18

12.6.2. SPEECH AND COMMAND	19

12.6.3.THE RELATIONSHIP BETWEEN SPEECH AND ADDRESS	19

12.6.4. THREE-LINE SERIAL CONTROL I/O EXTENSION OUTPUT	19

12.6.5. THREE-LINE SERIAL CONTROL TIMING	20

12.6.6. PROGRAM EXSAMPLE	20

13. CONTROL TIMING	21

13.1.CONTROL TIMING DIAGRAM	21

13.2. ONE-LINE SERIAL CONTROL PROGRAMMING	21

13.3. ONE-LINE SERIAL CONTROL “C” PROGRAMMING LANGUAGE	22

13.4. THREE-LINE SERIAL COTROL PROGRAMMING	24

13.5. THREE-LINE SERIAL CONTROL “C” PROGRAMMING LANGUAGE	25

14.VERSION HISTORY	27

# 1. FEATURES OF VOICE CHIPS

* Packages: DIP16, DIP28, DIP18, SSOP20, LQFP32, etc.

* Duration depends on the SPI flash size, support 2M-32M SPI flash.

* Built-in high-speed audio processor.

* Built-in 13Bit D/A converter, 12Bit PWM output with excellent timbre.

* Sampling frequency 6KHz,8KHz,10KHz,12KHz,14KHz,16KHz,18KHz 20KHz.

* PWM output can direct drive 0.5W/8Ωspeaker.

* Support DAC/PWM two kinds of output.

* Support mp3/wav audio files.

* Download by USB port, support online and offline download, even the WT588D is power on, data also can be loaded to SPI flash.

* Support mp3 control mode, key control mode, one- line serial control mode, and three-line serial control mode

* Set the BUSY signal output mode which shows voice playing status optionally.

* Equip with WT588D voice chip PC software, make its functions more outstanding.

* Accomplish setting control modes, voice combination, transfer voice, and insert mute, etc.

* Mute duration can be inserted between 10 ms to 25 minutes

* Strong anti-interference, can be used in the industrial field.

* Enter into sleep mode immediately and waste low power after stop playing.

* Mute duration can be inserted between 10ms to 25minutes,will not take any memory capacity.

* Strong anti-interference, can be used in the industrial field.

* Enter into sleep mode immediately and waste low power after stop playing

*15 kinds of key control mode, one button can set one control mode.

* Load 500 phrases voice max for editing.

* Support MIDI.

* Working voltage: DC2.8V~5V

* Standy current: <10uA

# 2. WT588D FUNCTION DESCRIPTIONS

WT588D is a powerful re-programming voice chip microcontroller. Its control mode can be changed and download data to SPI flash by our PC software. This software is easy operation, support online download, even the chip in power on , data can be download to the related SPI flash then reset the circuit, the control mode will be updated.

In the Mp3 control mode, this chip with mp3 player functions, such as play, pause, previous, next, volume up, volume down.

In the key control mode, the trigger mode is flexible, any key can be set as edge retrigger, edge no retrigger, level unloop, level loop, level hold loop, prev unloop, next unloop, prev loop, next loop, no function, pause, stop, vol+, vol- and on/off  15 kinds of trigger mode, control 10 key trigger output max.

In the 3x8 key combination control mode, 24 addresses’ voice can be triggered by edge retrigger mode.

In parallel interface control mode, 8 I/O port can be used in controlling max

In one-line serial interface control mode, voice play, stop, re-play, volume adjustment can be controlled by MCU.

Three-line serial interface control mode and three-line serial interface control I/O port expand mode can be switched , in the three-line control mode, voice play, stop, re-play and volume adjustment can be controlled , or direct trigger voice in 0-219 addresses. In the three-line serial control I/O port extension mode, can extend 8 bit output.

Switch between these two control mode, last working status still keep going in current mode.

PWM output can direct drive 0.5W/8Ω speaker, DAC output connect to external amplifier.

WT588D with a wide range of applications, such as bus reporter, alarm, reminder, alarm clock, study apparatus, household appliances, medical instrument, electronic toys, telecommunication, parking distance control and so on.

# 3. WT588D SELECTION

| MODEL | FLASH | I/O | PACKAGE | KEY MODE | PARALLEL MODE | 3X8 MATRIX MODE | MP3 MODE | ONE-LINE SERIAL MODE | THREE-LINE SERIAL MODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WTW500-16 | 16M | 4 | 16PIN MODULE | OK | ------------- | ----------- | OK | OK | OK |
| WTW500-28 | 16M | 11 | 28PIN MODULE | OK | OK | OK | OK | OK | OK |
| WT588D-18P | EXTERNAL | 4 | DIP18 | OK | ------------- | ---------- | OK | OK | OK |
| WT588D2SS | EXTERNAL | 4 | SSOP20 | OK | ------------- | ---------- | OK | OK | OK |
| WT588D-32L | EXTERNAL | 11 | LQFP32 | OK | OK | OK | OK | OK | OK |

# 4. THE RELATION BETWEEN FLASH CAPACITY AND DURATION

Sampling frequency at 6KHZ

| CAPACITY | 2M | 4M | 8M | 16M | 32M |
| --- | --- | --- | --- | --- | --- |
| TIME | 30sec | 100sec | 200sec | 500sec | 1000sec |

# 5. APPLICATIONS

# 6. WT588D APPLICATION DIAGRAM

BUSY output

USB download

Control interface

WT588D voice chips &

SPI-Flash

Audio output

# 7. PACKAGE SKETCH

16PIN MODULE                 28PIN MODULE (V1.1)             28PIN MODULE (V1.2)

LQPF32 CHIP                       DIP18 CHIP                     SSOP20 CHIP

# 8. PIN DESCRIPTIONS

## 8.1. WTW-16P PIN’S DESCRIPTION

| PIN | MARK | BRIET DESCRIPTION | FUNCTION |
| --- | --- | --- | --- |
| 1 | RESET | RESET | RESET PIN |
| 2 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 3 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 4 | PWM- | PWM- | PWM- AUDIO OUTPUT |
| 5 | P14 | DI | PROGRAMMING INPUT |
| 6 | P13 | DO | PROGRAMMING OUTPUT |
| 7 | P16 | CLK | CLOCK |
| 8 | GND | GND | GROUND |
| 9 | P15 | CS | PROGRAMMING CHIP-SELECT |
| 10 | P03 | K4/CLK/DATA | KEY/THREE-LINE CLOCK/ONE-LINE DATA INPUT |
| 11 | P02 | K3/CS | KEY/THREE-LINE CHIP-SELECT INPUT |
| 12 | P01 | K2/DATA | KEY/THREE-LINE DATA INPUT |
| 13 | P00 | K1 | KEY INPUT |
| 14 | VCC | VCC | MEMORIZER POWER INPUT |
| 15 | BUSY | BUSY | PLAY BUSY SIGNAL OUTPUT |
| 16 | VDD | VDD | DIGITAL POWER INPUT |

## 8.2. WTW-28P (V1.1) PIN’S DESCRIPTION

| PIN | MARK | BRIEF DESCRIPTION | FUNCTION |
| --- | --- | --- | --- |
| 1 | NC | NC | NC |
| 2 | NC | NC | NC |
| 3 | NC | NC | NC |
| 4 | P10 | K9/A7/R1 | KEY/PARALLEL ADDRESS/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION ADDRESS |
| 5 | P11 | K10/R2 | KEY/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION ADDRESS |
| 6 | P12 | R3 | MATRIX ROW INPUT/THREE-LINE EXTENSION ADDRESS |
| 7 | RESET |  | RESET |
| 8 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 9 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 10 | PWM- | PWM- | PWM- AUDIO OUTPUT |
| 11 | P14 | DI | PROGRAMMING DATA INPUT |
| 12 | P13 | DO | PROGRAMMING DATA OUTPUT |
| 13 | P16 | CLK | PROGRAMMING CLOCK |
| 14 | GND | GND | GROUND |
| 15 | P15 | CS | PROGRAMMING CHIP-SELECT |
| 16 | P03 | K4/A2/L3/CLK/DATA | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THEREE-LINE CLOCK/ONE-LINE DATA INPUT |
| 17 | P02 | K3/A1/L2/CS | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THREE-LINE CHIP-SELECT INPUT |
| 18 | P01 | K2/A0/L1/DATA | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THREE-LINE DATA INPUT |
| 19 | P00 | K1/L0/SBT | KEY/MATRIX COLUMN/SBT PARALLEL ADDRESS TRIGGER OUTPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 20 | VCC | VCC | ANOLOG POWER INPUT |
| 21 | BUSY | BUSY | PLAY BUSY SIGNAL OUTPUT |
| 22 | VDD | VDD | DIGITAL POWER INPUT |
| 23 | P04 | K5/A3/L4 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 24 | P05 | K6/A4/L5 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 25 | P06 | K7/A5/L6 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 26 | P07 | K8/A6/L7 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 27 | NC | NC | NC |
| 28 | NC | NC | NC |

## 8.3. WTW-28P (V1.2) PIN’S DESCRIPTION

| PIN | MARK | BRIEF DESCRIPTION | FUNCTION |
| --- | --- | --- | --- |
| 1 | P05 | K6/A4/L5 | KEY/PARALLEL ADDRESS/MATRIX ROW INPUT/THREE-LINESERIAL EXTENSION ADDRESS |
| 2 | P06 | K7/A5/L6 | KEY/PARALLEL ADDRESS/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION ADDRESS |
| 3 | P07 | K8/A6/L7 | KEY/PARALLEL ADDRESS/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION ADDRESS |
| 4 | P10 | K9/A7/R1 | KEY/PARALLEL ADDRESS/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION ADDRESS |
| 5 | P11 | K10/R2 | KEY/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION ADDRESS |
| 6 | P12 | R3 | KEY/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION ADDRESS |
| 7 | RESET | RESET | RESET |
| 8 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 9 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 10 | PWM- | PWM- | PWM- AUDIO OUTPUT |
| 11 | P14 | DI | PROGRAMMING DATA INPUT |
| 12 | P13 | DO | PROGRAMMING DATA OUTPUT |
| 13 | P16 | CLK | PROGRAMMING CLOCK |
| 14 | GND | GND | GROUND PIN |
| 15 | P15 | CS | PROGRAMMING CHIP-SELECT |
| 16 | P03 | K4/A2/L3/CLK/DATA | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THEREE-LINE CLOCK/ONE-LINE DATA INPUT |
| 17 | P02 | K3/A1/L2/CS | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THREE-LINE CHIP-SELECT INPUT |
| 18 | P01 | K2/A0/L1/DATA | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THREE-LINE CHIP-SELECT INPUT |
| 19 | P00 | K1/L0/SBT | KEY/MATRIX COLUMN/SBT PARALLEL ADDRESS TRIGGER OUTPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 20 | VCC | VCC | ANOLOG POWER INPUT |
| 21 | BUSY | BUSY | PLAY BUSY SIGNAL OUTPUT |
| 22 | VDD | VDD | DIGITAL POWER INPUT |
| 23 | P04 | K5/A3/L4 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 24 | NC | NC | NC |
| 25 | NC | NC | NC |
| 26 | NC | NC | NC |
| 27 | NC | NC | NC |
| 28 | NC | NC | NC |

## 8.4. WT588D-32L PACKAGE PIN’S DESCRIPTION

| PIN | MARK | BRIEF DESCRIPTION | FUNCTION |
| --- | --- | --- | --- |
| 1 | P14 | DI | MEMORIZER DATA COMMUNICATIONS INPUT |
| 2 | P15 | DO | MEMORIZER COMMUNICATIONS CHIP-SELECT |
| 3 | P16 | CS | MEMORIZER COMMUNICATIONS CLOCK |
| 4 | VDD-SIM | CLK | SERIAL POWER INPUT |
| 5 | P00 | VDD-SIM | KEY/MATRIX COLUMN/SBT PARALLEL ADDRESS TRIGGER OUTPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 6 | P01 | K2/A0/L1/DATA | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THREE-LINE DATA INPUT |
| 7 | P02 | K3/A1/L2/CS | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THREE-LINE CHIP-SELECT INPUT |
| 8 | P03 | K4/A2/L3/CLK/DATA | KEY/PARALLEL ADDRESS/MATRIX COLUMN/THREE-LINE CLOCK/ONE-LINE DATA INPUT |
| 9 | P04 | K5/A3/L4 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 10 | P05 | K6/A4/L5 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 11 | P06 | K7/A5/L6 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 12 | P07 | K8/A6/L7 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 13 | P10 | K9/A7/R1 | KEY/PARALLEL ADDRESS/MATRIX COLUMN INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 14 | P11 | K10/R2 | KEY/MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 15 | P12 | R3 | MATRIX ROW INPUT/THREE-LINE SERIAL EXTENSION OUTPUT ADDRESS |
| 16 | VDD | VDD | CHIP POWER INPUT |
| 17 | P17 | BUSY | PLAY BUSY SIGNAL OUTPUT |
| 18 | CVDD | CVDD | VDD POWER VOLTAGE ADJUSTMENT |
| 19 | OSCI | OSCI | CRYSTAL OSCILLATOR INPUT |
| 20 | RESET | RESET | RESET |
| 21 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 22 | VDD-SPK | VDD-SPK | AUDIO POWER INPUT |
| 23 | PWM- | PWM- | PWM- AUDIO OUTPUT |
| 24 | VSS-SPK | VSS-SPK | AUDIO POWER GROUND |
| 25 | VSS | VSS | GROUND |
| 26 | NC | NC | NC |
| 27 | NC | NC | NC |
| 28 | NC | NC | NC |
| 29 | NC | NC | NC |
| 30 | NC | NC | NC |
| 31 | NC | NC | NC |
| 32 | P13 | DO | MEMORIZER DATA COMMUNICATIONS OUTPUT |

## 8.5. WT588D-18P PACKAGE PIN’S DESCRIPTION

| PIN | MARK | DESCRIPTION | FUNCTION |
| --- | --- | --- | --- |
| 1 | P00 | K1 | KEY |
| 2 | P01 | K2/DATA | KEY/ THREE-LINE DATA INPUT |
| 3 | P02 | K3/CS | KEY/ THREE-LINE CS INPUT |
| 4 | P03 | K4/A2/CLK/DATA | KEY/PARALLEL ADDRESS/THREE-LINE CLOCK/ONE-LINE DATA INPUT |
| 5 | VDD | VDD | POWER IN |
| 6 | P17 | BUSY | BUSY OUT |
| 7 | CVDD | CVDD | VDD POWER ADJUSTMENT |
| 8 | OSCI | OSCI | CRYSTAL OSCILLAOR INPUT |
| 9 | RESET | RESET | RESET, KEEP LOW LEVEL>5MS |
| 10 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 11 | VDD-SPK | VDD-SPK | AUDIO POWER INPUT |
| 12 | PMW- | PWM- | PWM- AUDIO OUTPUT |
| 13 | VSS | VSS | GROUND |
| 14 | P13 | DO | SPI-FLASH COMMUNICATION DATA OUTPUT |
| 15 | P14 | DI | SPI-FLASH COMMUNICATION DATA INPUT |
| 16 | P15 | CS | SPI-FLASH COMMUNICATION CS |
| 17 | P16 | CLK | SPI-FLASH COMMUNICATION CLOCK |
| 18 | VDD-SIM | VDD-SIM | SERIALINTERFACE POWER MANAGEMENT |

## 8.6. WT588D-20SS PACKAGE PIN’S DESCRIPTION

| PIN | MARK | DESCRIPTION | FUNCTION |
| --- | --- | --- | --- |
| 1 | P13 | DO | SPI-FLASH DATA COMMUNICATIONS OUTPUT |
| 2 | P14 | DI | SPI-FLASH DATA COMMUNICATIONS INPUT |
| 3 | P15 | CS | SPI-FLASH COMMUNICATIONS CHIP-SELECT |
| 4 | P16 | CLK | SPI-FLASH COMMUNICATIONS CLOCK |
| 5 | VDD-SIM | VDD-SIM | SERIAL INTERFACE POWER MANAGEMENT |
| 6 | P00 | K1 | KEY |
| 7 | P01 | K2/DATA | KEY /THREE-LINE DATA INPUT |
| 8 | P02 | K3/CS | KEY /THREE-LINE CHIP-SELECT INPUT |
| 9 | P03 | K4/ CLK/DATA | KEY /THREE-LINE CLOCK/ONE-LINE DATA INPUT |
| 10 | VDD | VDD | POWER INPUT |
| 11 | P17 | BUSY | PLAY BUSY SIGNAL OUTPUT |
| 12 | CVDD | CVDD | VDD POWER ADJUSTMENT |
| 13 | OSCI | OSCI | CRYSTAL OSCILLATOR INPUT |
| 14 | RESET | RESET | RESET, KEEP LOW LEVEL>5MS |
| 15 | VSS | VSS | GROUND |
| 16 | PWM+/DAC | PWM+/DAC | PWM+/DAC AUDIO OUTPUT |
| 17 | VDD-SPK | VDD-SPK | AUDIO POWER INPUT |
| 18 | PWM- | PWM- | PWM- AUDIO INPUT |
| 19 | VSS-SPK | VSS-SPK | AUDIO POWER GROUND |
| 20 | NC | NC | NC |

# 9. PIN’S DETAILED DESCRIPTION

THE USE OF K1－K10

K1－K10 defined as I/O port of key trigger ,corresponding I/O port P00－p07|、p10 and P11. Switch I/O port level by K1 －k10 or input edge to I/O port, trigger to play the voice. Trigger mode of Key K0－k9 can be setted as edge retrigger, edge no retrigger, level unloop, level loop, , level hold loop, prev unloop, next unloop, prev loop, next loop, no function, pause, stop, vol+, vol- and on/off 15 kinds of trigger mode.

KEY MODE: P00→K1    P01→K2     P02→K3     P03→K4      P04→K5

P05→K6    P06→K7     P07→K8     P10→K9      P11→K10

ONE-LINE SERIAL INTERFACE MODE:  P00→K1   P04→K5   P07→K8

P01→K2   P05→K6   P10→K9

P02→K3   P06→K7   P11→K10

THE USE OF R1-R3、L0－L7

In the key control mode of 3x8 matrix, I/O port defined as R－R3、L0－L7，R1－R3 as matrix row input , L0－L7 as matrix column input. A phrase can be triggered and play when short circuit R and L. In this control mode, all keys trigger mode were defined as EDGE RETRIGGER. The details please find 12.3

The use of SBT and A0-A7

In the parallel control mode, I/O port P00-P07、P10 defined as SBT, A0-A7.  SBT is audition feet, A0-A7 are address, from low to high. I/O port details please find 12.4

# 10. ELECTRONIC PARAMETERS

（VDD − VSS = 4.5V, TA = 25°C，without load）

| PARAMETER | MARK | ENVIRONMENT CONDITION | MIN VALUE | TYPICAL VALUE | MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| OPERATING VOLTAGE | VDD | Fsys=8MHz | 2.8 |  | 5.5 | V |
| OPERATING CURRENT | IOP1 | WITHOUT LOAD | - | 4.5 | 5.5 | mA |
| SUSPEND CURRENT | IDD2 | WITHOUT LOAD | - | 1 | 3 | uA |
| HIBERNATE MODE | IOP2 | WITHOUT LOAD | - | 650 |  | uA |
| LOW VOLTAGE INPUT | VIL | ALL OF PINS INPUT | VSS | - | 0.3VDD | V |
| HIGH VOLTAGE INPUT | VIH | ALL OF PINS INPUT | 0.7VDD | - | VDD | V |
| CURRENT INPUT BP1、BP2、RESET | IIN1 | VIN=0V PULL RESISTANCE=500KΩ | -5 | -9 | -14 | uA |
| CURRENT INPUT BP1、BP2、RESET | IIN2 | VIN=0V PULL RESISTANCE=150KΩ | -15 | -30 | -45 | uA |
| CURRENT INPUT（BP0） | IOL | VDD=3V，VOUT=0.4V | 8 | 12 | - | mA |
|  | IOH | VDD=3V，VOUT=2.6V | -4 | -6 | - | mA |
|  | IOL | VDD=4.5V，VOUT=1.0V | - | 25 | - | mA |
|  | IOH | VDD=4.5V，VOUT=2.6V | - | -12 | - | mA |
| CURRENT OUTPUT（BP1） | IOL | VDD=3V，VOUT=0.4V | 4 | 10 | - | mA |
|  | IOH | VDD=3V，VOUT=2.6V | -4 | -6 | - | mA |
| CURRENT OUTPUT PWM+/PWM- | IOL1 | RL=8Ω 【PWM+】--【RL】--【PWM-】 | +200 | - | - | mA |
|  | IOH1 |  | -200 | - | - | mA |
| DAC MAX CURRENT | IDAC | RL=100Ω | -2.4 -4.0 | -3.0 -5.0 | -3.6 -6.0 | mA |
| PULL RESISTANCE TASTING | RPL |  | 75 | 150 | 225 |  |

# 11. ENVIRONMENTAL LIMITED PARAMETER

| PARAMETER | MARK | CONDITION | RATED VALUE | UNIT |
| --- | --- | --- | --- | --- |
| POWER | VDD — VSS | - | -0.3～+7.0 | V |
| INPUT VOLTAGE | VIN | ALL INPUT | VSS-0.3～VDD+0.3 | V |
| STORE TEMPERATURE | TSTG | - | -55～+150 | °C |
| USING TEMPERATURE | TOPR | - | -40～+85 | °C |

# 12. CONTROL MODE

## 12.1. MP3 MODE

In the mp3 mode, default function of 6 I/O port

| I/O | P00 | P01 | P02 | P03 | P04 | P05 |
| --- | --- | --- | --- | --- | --- | --- |
| FUNCTION | STOP | PLAY/PAUSE | NEXT | PREVIOUS | VOL+ | VOL- |

## 12.2. KEY CONTROL MODE

Defined pin can trigger a function of the chip, each pin’s trigger mode can be set solely. Anti-shake time of the key control mode is 10ms. In this mode, include edge retrigger, edge irretrigger, level unloop, level loop,  level hold loop, prev unloop, next unloop, prev loop, next loop, no function, pause, stop, vol+, vol- and on/off  15 kinds of trigger mode. Detailed diagram as follows:

### 12.2.1. EDGE RETRIGGER

Remark: Negative edge trigger. When the I/O port test falling edge (for example short circuit of I/O and GND at 10ms), the voice will be triggered and playing. If another falling edge is tested, the voice will be interrupted and replay.

### 12.2.2. EDGE IRRETRIGGER

Remark: Negative edge trigger. When falling edge was tested by I/O port (for example short circuit of I/O and GND at 10ms), the voice can be triggered and playing. During the voice are playing, the falling edge was tested again, the voice will not be interrupted until the voice is over.

### 12.2.3. LEVEL LOOP

Remark: Triggered by low level, mean press the button. It will play the voice when you press the button until you stop playing. If you keep to press all the time, it will keep playing, loop.

### 12.2.4. LEVEL UNLOOP

Remark: Triggered by low level. Mean press the button. It will play the voice when you press the button until you stop playing. But it just play one time, no loop, whatever you keep to press the button.

### 12.2.5. LEVEL HOLD LOOP

Remark: Negative edge/level trigger. When the I/O at low level(for example short circuit of I/O and GND at 10ms), it will play the voice one time, during the playing even turn to high level it will not stop until the first play is over. If still keep low level, it will keep playing again and again, loop.

### 12.2.6. ON/OFF

Remark: Negative edge trigger. Press the button it will play voice, and press again, the voice will stop.

### 12.2.7. NEXT UNLOOP

Remark: Negative edge trigger. Press the button it will play the next song, press again, plays the next, etc. It will not to play song if the last song is play over.

### 12.2.8. PREV UNLOOP

Remark: Negative edge trigger. Press the button it will play the previous song, press again, plays the pre-, etc. It will not to play song if the first song is play over.

### 12.2.9. NEXT LOOP

Remark: Negative edge trigger. Press the button it will play the next song, press again, plays the next, etc. It will play the first song if the last song is play over.

### 12.2.10. PREV LOOP

Remark: Negative edge trigger. Press the button it will play the previous song, press again, plays the pre-, etc. It will play the last song if the first song is play over.

### 12.2.11. PAUSE

Remark: Negative edge trigger. Press the button it will pause to play the current song, then press the button again, it will go to play the song.

### 12.2.12. STOP

Remark: Negative edge trigger. Press the “stop” button it will pause to play the current song, even if press the button which control to play the song, is invalid.

## 12.3. 3X8 MATRIX KEY CONTROL MODE

In this mode, all keys default as edge retrigger, can trigger 24 groups voice directly, voice addresses can be defined.

| I/O | P00 | P01 | P02 | P03 | P04 | P05 | P06 | P07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P10 | Key1 | Key2 | Key3 | Key4 | Key5 | Key6 | Key7 | Key8 |
| P11 | Key9 | Key10 | Key11 | Key12 | Key13 | Key14 | Key15 | Key16 |
| P12 | Key17 | Key18 | Key19 | Key20 | Key21 | Key22 | Key23 | Key24 |

## 12.4. PARALLEL CONTROL MODE

In this mode, P00 defined as SBT trigger feet, playing voice by changing the address “An”. Pins’ definition as follows:

| PACKAGES | PINS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P00 | P01 | P02 | P03 | P04 | P05 | P06 | P07 | P10 |
| WTW500-28 | SBT | A0 | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| WT588D-32L | SBT | A0 | A1 | A2 | A3 | A4 | A5 | A6 | A7 |

### 12.4.1. VOICE ADDRESSES CORRESPONDING LIST

| GROUP N (VOICE) | ADDRESS FEET |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A7 | A6 | A5 | A4 | A3 | A2 | A1 | A0 |
| GROUP0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| GROUP1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| GROUP2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| …… | …… | …… | …… | …… | …… | …… | …… | …… |
| GROUP217 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| GROUP218 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| GROUP219 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |

### 12.4.2. CONTROL TIMING DIAGRAM

Set the addresses to I/O(P01～P10) by MCU, then play the speech by trigger the SBT feet , the trigger mode can be defined as edge retrigger, edge on retrigger, on/off, prev unloop, next unloop, prev loop, next loop, level unloop, level loop, level hold loop, pause.

10MS

10MS

Send addresses, then send SBT trigger playing voice after 2ms, and signal maintain 10ms.

## 12.5. ONE-LINE SERIES INTERFACE CONTROL MODE

In this mode, can achieve functions: ON / OFF, Volume adjustable, Trigger playing voice, etc. The keys from P00-P10 can choose any trigger mode. Note: In this mode, the voice chip without sleep state, after stopping play, the current is about 5mA, please take care when use battery supply.

### 12.5.1. ASSIGNMENT OF I/O

| PACKAGES | I/O PORTS OF VOCIE CHIPS OR MODULES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P00 | P01 | P02 | P03 | P04 | P05 | P06 | P07 | P10 | P11 |
| WT588D-18P | K1 | K2 | K3 | DATA | --- | --- | --- | --- | --- | --- |
| WT588D-20SS | K1 | K2 | K3 | DATA | --- | --- | --- | --- | --- | --- |
| WT588D-32L | K1 | K2 | K3 | DATA | K5 | K6 | K7 | K8 | K9 | K10 |
| WTW500-16 | K1 | K2 | K3 | DATA | --- | --- | --- | --- | --- | --- |
| WTW500-28 | K1 | K2 | K3 | DATA | K5 | K6 | K7 | K8 | K9 | K10 |

### 12.5.2. COMMAND AND CODE

| CODE | FUNCTIONS | DESCRIPTIONS |
| --- | --- | --- |
| E0H～E7H | VOLUME ADJUSTMENT | DURING THE PLAYING OR STANDBY,THERE ARE 8 LEVEL VOLUME CAN BE ADJUSTED BY SENDING COMMAND, E0H IS MINIMUM, E7H IS MAXIMUM |
| F2H | LOOP PLAY | DURING THE PLAY, SPEECH IN CURRENT ADDRESS CAN BE LOOP PALYED BY SENDING COMMAND |
| FEH | STOP TO PLAY | STOP PLAYING |

### 12.5.3. THE RELATIONSHIP BETWEEN SPEECH AND ADDRESS

| DATA(HEX) | FUNCTION |
| --- | --- |
| 00H | PLAY VOICE OF GROUP 0 |
| 01H | PLAY VOICE OF GROUP 1 |
| 02H | PLAY VOICE OF GROUP 2 |
| ….. | ……. |
| D9H | PLAY VOICE OF GROUP 217 |
| DAH | PLAY VOICE OF GROUP 218 |
| DBH | PLAY VOICE OF GROUP 219 |

### 12.5.4. CONTROL TIMING DIAGRAM

One Line Serial mode only need one line, the data bit is depend on the duty cycle. Pull low RESET signal 5ms, wait not less than 17ms in high level, and pull low data signal 5ms before sent out data. The data bit is 0 if duty cycle is 1/3 , and is 1 if duty cycle is 2/3.

Remark: Reset before sending the data, unless work in interferential environment. DATA is for one-line data communication. Pull low 5ms after reset , for waking up the chip, low level before high level.  2mS after data sent out, BUSY output will be acted.

### 12.2.5. ONE LINE SERIAL CONTROL DIAGRAM SAMPLE

FOR EXAMPLE, IN THE ONE-LINE CONTROL MODE, THE TIMING OF SENDING DATA 9CH

### 12.5.5. PROGRAM EXAMPLE

MICROCONTROLLER: PIC16F54, CLOCK SPEED:4MHz

Send oneline(unsigned char addr)

{

rst=0; /*reset the chip */

delay1ms(2); /*delay 1ms to 5ms */

rst=1;

delay1ms(6); /*delay 6ms */

sda=0;

delay1ms(5); /* delay 5ms */

for(i=0;i<8;i++)

{  sda=1;

if(addr & 1)

{ delay1us(600); /* 600us */

sda=0;

delay1us(300); }  /* 300us */

else {

delay1us(300); /* 300us */

sda=0;

delay1us(600); } /* 600us */

addr>>=1; }

sda=1; }

## 12.6.THREE-LINE SERIAL CONTROL MODE

This mode consist of three communication line, they are CS,DATA,SCK, timing imitate the standard SPI means of communication.

### 12.6.1.ASSIGNMENT OF I/O

| PACKAGE | I/O |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P00 | P01 | P02 | P03 | P04 | P05 | P06 | P07 | P10 | P11 |
| SOP28 | -- | DATA | CS | SCK | -- | -- | -- | -- | -- | -- |
| SSOP28 | -- | DATA | CS | SCK | -- | -- | -- | -- | -- | -- |

### 12.6.2. SPEECH AND COMMAND

| CODE | FUNCTION | DESCRIPTION |
| --- | --- | --- |
| E0H-E7H | VOLUME ADJUSTMENT | DURING THE PLAYING OR STANDBY,THERE ARE 8 LEVEL VOLUME CAN BE ADJUSTED BY SENDING COMMAND, E0H IS MINIMUM, E7H IS MAXIMUM |
| F2H | LOOP PLAY | DURING THE PLAY, SPEECH IN CURRENT ADDRESS CAN BE LOOP PALYED BY SENDING COMMAND |
| FEH | STOP TO PALY | THE COMMAND OF STOP PLAYING |
| F5H | ENTRY I/O EXTENSION OUTPUT | IN THE COMMOND THREE-LINE SERIAL MODE,ENTRY I/O EXTENSION OUTPUT STATE BY SENDING COMMAND |
| F6H | EXIT I/O EXTENSION OUTPUT | IN THE I/O EXTENSION OUTPUT STATE, ENTRY COMMOND THREE-LINE SERIAL MODE BY SENDING THIS COMMAND |

### 12.6.3.THE RELATIONSHIP BETWEEN SPEECH AND ADDRESS

| DATA(HEX) | FUNCTION |
| --- | --- |
| 00H | PLAY THE SPEECH OF 0 |
| 01H | PLAY THE SPEECH OF 1 |
| 02H | PLAY THE SPEECH OF 2 |
| ….. | ………. |
| D9H | PLAY THE SPEECH OF 217 |
| DAH | PLAY THE SPEECH OF 218 |
| DBH | PLAY THE SPEECH OF 219 |

### 12.6.4. THREE-LINE SERIAL CONTROL I/O EXTENSION OUTPUT

In the three-line serial control mode, entry three-line serial I/O extension output by sending data F5H, sending the binary data and make the related I/O to output high level ,sequentially, control the periphery circuit . Send data F6H,will exit this mode, entry commond three-line serial control mode.

| GROUD N | I/O |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P12 | P11 | P10 | P07 | P06 | P05 | P04 | P00 |
| GROUP 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| GROUP 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| GROUP 2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| ………… | ………… | ………… | ………… | ………… | ………… | ………… | ………… | ………… |
| GROUP217 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| GROUP218 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| GROUP219 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |

The data in the form, 0 represents low level output, 1 represents high level output. Switch to I/O extension output from three-line serial control mode, the last trigger mode of three-line serial control mode be kept. If setted loop play, the last triggered speech will keep loop playing until switch back to three-line serial mode, and change trigger mode. Switch I/O extension mode to three-line serial control mode, the last I/o extension function also be kept, in the I/O extension output,P00 setted as high level output, after switch to three-line serial control mode, P00 still keep high level output until switch back to I/O extension output and change settings.

I/O extension output can extend 8 I/O,  applicate in the situation of poor I/O.

### 12.6.5. THREE-LINE SERIAL CONTROL TIMING

Three-line serial control mode consist of CS, SCK , DATA, timing imitate standard SPI communication mode, pull low 1ms-5ms before reset signal send out code. During working , RESET pin should keep high when chip  is working. CS should keep low 2ms-10ms to wake up the chip,data LSB first,data clock in chip in rising edge. clock cycle is 40us-2ms. BUSY signal will active in 2ms after data transmit success. Low first ,then high ,when transmit data . Timing diagram as follow:

### 12.6.6. PROGRAM EXSAMPLE

(Microcotroller PIC16F54,system speed 4MHz)

Send threelines(unsigned char addr)

｛rst=0;

delay1ms(5);rst=1;

delay1ms(17);

cs=0;

delay1ms(2);

for(i=0;i<8;i++)

｛	scl=0;

if(addr & 1)sda=1;

else sda=0;

addr>>=1;

delay1us(30);

scl=1;

delay1us(30);	｝

cs=1;｝

# 13. CONTROL TIMING

## 13.1.CONTROL TIMING DIAGRAM

## 13.2. ONE-LINE SERIAL CONTROL PROGRAMMING

This program only for one-line serial control mode application circuit in “WT588D CHIP&MODULE DETAILED INFORMATION”.

ORG 0000H

KEY EQU P1.1    ;key pin

RST EQU P1.4    ;reset pin

SDA EQU P3.0    ;data pin

DAIFAZHI EQU 50H     ;send code value temp

MOV DAIFAZHI,#0H;intial send value 0

MOV R5,#8       ;loop eight times

MAIN:

JB KEY,MAIN

MOV R6,#20      ;delay 20MS

LCALL DELAY1MS

JB KEY,MAIN     ;key press debounce

JNB KEY,$       ;wait key pressed release

LCALL one_line  ;call one line program

INC DAIFAZHI    ;send value add 1

MOV A,DAIFAZHI

CJNE A,#210,XX2 ;

XX2: JC XX3

MOV DAIFAZHI,#0H

XX3: LJMP MAIN

one_line:           ;////one line send code subprogram

CLR RST

MOV R6,#5     ;delay5MS

LCALL DELAY1MS

SETB RST

MOV R6,#17     ;delay17MS

LCALL DELAY1MS

CLR SDA

MOV R6,#5     ;delay5MS

LCALL DELAY1MS

MOV A,DAIFAZHI

LOOP:    SETB SDA

RRC A

JNC DIDIANPIN  ;high level   high:low=2:1

LCALL DELAY200US

LCALL DELAY200US

CLR  SDA

LCALL DELAY200US

LJMP LOOP1

DIDIANPIN:                ;low level  low:high=1:2

LCALL DELAY200US

CLR  SDA

LCALL DELAY200US

LCALL DELAY200US

LOOP1:  DJNZ R5,LOOP

MOV R5,#08H

SETB SDA

RET

DELAY200US:  MOV R6,#100         ;delay 400US subprogram

DJNZ R6,$

RET

DELAY1MS:                	 ;delay 1mssub program, change r6 value can change delay time

L1:  MOV R7,#248

DJNZ R7,$

DJNZ R6,L1

RET

END

## 13.3. ONE-LINE SERIAL CONTROL “C” PROGRAMMING LANGUAGE

This program only for one-line serial control mode application circuit in “WT588D CHIP&MODULE DETAILED INFORMATION”.

#include <at89x2051.H>

sbit KEY=P1^1; /*P1_1 is P1 bit2*/

sbit RST=P1^4^; /*P1_4 is bit3*/

sbit SDA=P3^0; /*P3_0  is bit4*/

void delay1ms(unsigned char count) //1MS delay subprogram

{

unsigned char i,j,k;

for(k=count;k>0;k--)

for(i=2;i>0;i--)

for(j=248;j>0;j--);

}

void delay100us(unsigned char count)  //100US delay subprogram

{ unsigned char i;

unsigned char j;

for(i=count;i>0;i--)

for(j=50;j>0;j--);

}

Send_oneline(unsigned char addr)

{

unsigned char i;

RST=0;

delay1ms(5);	    	/*wait reset delay 5MS*/

RST=1;

delay1ms(17);         /* delay 17ms */

SDA=0;

delay1ms(5);         /* delay 5ms */

for(i=0;i<8;i++)

{SDA=1;

if(addr & 1)

{delay100us(4);       /* 400us */

SDA=0;

delay100us(2);       /* 200us */

}

else {

delay100us(2); 	   /* 200us */

SDA=0;

delay100us(4);       /* 400us */

}

addr>>=1; }

SDA=1;

}

main()

{unsigned char FD=0;

P3=0XFF;

while(1)

{

if(KEY==0)

{

delay1ms(10);

if(KEY==0)     //press key P1.1 increase by decrease key value

{

Send_oneline(FD);

FD++;

if(FD==210) //one line control mode, control 210 sectors at best

{

FD=0;

}

while(KEY==0);  //wait key pressed release, debounce time

}

}

}

}

## 13.4. THREE-LINE SERIAL COTROL PROGRAMMING

This program only for three-line serial control mode application circuit in “WT588D CHIP&MODULE DETAILED INFORMATION”.

ORG 0000H

KEY EQU P1.1    ;key pin

RST EQU P1.4    ;reset pin

CS  EQU P3.1    ;CS trigger pin

SCL EQU P3.2    ;clock pin

SDA EQU P3.0    ;data pin

DAIFAZHI EQU 50H     ;send value temp address

MOV DAIFAZHI,#0H;intinal send value 0

MOV R5,#8       ;loop eight times

MAIN:

JB KEY,MAIN

MOV R6,#20     ;delay 20MS

LCALL DELAY1MS

JB KEY,MAIN     ;key debounce

JNB KEY,$       ;wait key pressed release

LCALL THREE_LINE;call three line subprogram

INC DAIFAZHI    ;send value add 1

MOV A,DAIFAZHI

CJNE A,#220,XX2 ;judge the sector address if at max

XX2: JC XX3

MOV DAIFAZHI,#0H

XX3: LJMP MAIN

THREE_LINE:           ;////three line subprogram

CLR RST

MOV R6,#5     ;delay 5MS

LCALL DELAY1MS

SETB RST

MOV R6,#17     ;delay 17MS

LCALL DELAY1MS

CLR CS

MOV R6,#2     ;delay 2MS

LCALL DELAY1MS

MOV A,DAIFAZHI

LOOP:

CLR SCL

RRC A

MOV SDA,C

LCALL DELAY50US

SETB SCL

LCALL DELAY50US

DJNZ R5,LOOP

MOV R5,#08H

SETB CS

RET

DELAY50US:    MOV R6,#25         ;delay 50US subprogram

DJNZ R6,$

RET

DELAY1MS:               	 ;delay 1ms,change r6 value can change delay time

L1:	      MOV R7,#248

DJNZ R7,$

DJNZ R6,L1

RET

END

## 13.5. THREE-LINE SERIAL CONTROL “C” PROGRAMMING LANGUAGE

This program only for three-line serial control mode application circuit in “WT588D CHIP&MODULE DETAILED INFORMATION”.

#include <at89x51.H>

sbit KEY=P1^1; /*P1_1 P1 bit2*/

sbit RST=P1^4; /*P1_4 P1 bit3*/

sbit  CS=P3^1; /*P3_1 P1 bit4 */

sbit SCL=P3^2; /*P3_2 P3 bit3*/

sbit SDA=P3^0; /*P3_0 P3 bi1*/

//sbit DENG=P3^7; /*P3_5 P3 bit6*/

void delay1ms(unsigned char count) //1MS  delay subprogram

{

unsigned char i,j,k;

for(k=count;k>0;k--)

for(i=2;i>0;i--)

for(j=248;j>0;j--);

}

void delay100us(void)  //100US delay subprogram

{

unsigned char j;

for(j=50;j>0;j--);

}

Send_threelines(unsigned char addr) //three line send code subprogram

{unsigned char i;

RST=0;

delay1ms(5);

RST=1;

delay1ms(17);         /* 17ms*/

CS=0;

delay1ms(2);

for(i=0;i<8;i++)

{SCL=0;

if(addr & 1)SDA=1;

else SDA=0;

addr>>=1;

delay100us();  /* 100us */

SCL=1;

delay100us();

}

CS=1;

}

main()

{unsigned char FD=0;

P3=0XFF;

while(1)

{

if(KEY==0)

{

delay1ms(20);

if(KEY==0)    // press key P1.1 increase by decrease key value

{

Send_threelines(FD);

FD++;

if(FD==220// three line control mode, control 220 sectors at best             	   {

FD=0;

}

while(KEY==0);  //wait key pressed release

}

}

}

}